Week 1–3 Deliverable 4

Gravitational Torques of the Hanging Planar Five-Bar

Bo Zhang (8571260)  |  Dr Chin-Hsing Kuo  |  September 2026

1. What was done

The unbalanced gravitational loads were derived for the frozen geometry and mid-link COM assumption. Because the five-bar is a closed chain, each actuator torque is not “this leg’s own weight only”. The correct statement is virtual work: T_w = ∂V/∂θ. If the cranks are left free, the force F at E that holds the mechanism satisfies J^T F = T_w. That F is what the IMADA PS-10N would read in a slow hold, before any GSM is fitted. No springs, gears, or sensors are used this week.

2. Potential and holding torque

With +y upward and datum on the ground line:

V = g ( m_a y_aL + m_a y_aR + m_b y_bL + m_b y_bR + m_E y_E )

Static motor torques that hold the pose:  T_w = (T_wL, T_wR) = ∂V/∂(θ_L, θ_R).

COM locations: crank COM at mid-crank; coupler COM at mid-coupler; payload at E. Masses from the CAD table: m_a = 40 g, m_b = 50 g, m_E = 300 g. E(θ) comes from the Week 1–3 forward kinematics. ∂E/∂θ is the 2×2 Jacobian Ė = J θ̇ obtained by differentiating the two coupler-length constraints.

3. Force at E (matches the later mechanical gauge)

If the two cranks are free (no motor torque), equilibrium is J^T F = T_w. F = (F_x, F_y) is the external force that must be applied at E. On the midline, F_x = 0 and F_y is upward (positive), about 3.6–3.7 N for the present masses — inside the 10 N PS-10N range. This is much smaller than “lift the entire 0.48 kg as a free mass” (4.7 N), because part of the link weight is carried by the grounded joints A and B.

4. Checkpoint values

| Pose | E (mm) | T_wL (mN·m) | T_wR (mN·m) | F_x (N) | F_y (N) |
| --- | --- | --- | --- | --- | --- |
| C1 CAD sample | (0, -180) | -475.49 | 475.49 | 0.000 | 3.672 |
| C2 midline high (safe) | (0, -120) | -876.56 | 876.56 | 0.000 | 3.589 |
| C3 midline low | (0, -250) | -244.51 | 244.51 | -0.000 | 3.632 |
| C4 left | (-60, -180) | -462.98 | 350.13 | -0.233 | 3.727 |
| C5 right | (60, -180) | -350.13 | 462.98 | 0.233 | 3.727 |

On the midline, T_wL = −T_wR and F_x = 0 (mirror). C4 and C5 swap and flip the torques: T_wL(C4) = −T_wR(C5). Analytic T_w matches a finite-difference of V(θ) to about 10^{-8}–10^{-9} N·m on these poses.

5. Singularity to stay away from

Near y ≈ −80 mm on the midline the two couplers line up (forward / Type-II singularity). The Jacobian is ill-conditioned and the motor torques blow up, even though F_y stays finite. The IK checkpoint at (0, −80) mm is therefore not used for Tw design. C2 is taken at (0, −120) mm instead. Later trajectories should stay below about y = −100 mm.

6. Midline curves

Figure 1. Unbalanced T_w and the hold force at E along x = 0 (y from −260 to −100 mm).

7. How to run

python3 scripts/test_five_bar_tw.py

python3 scripts/verify_five_bar_tw.py

from five_bar_statics import gravity_loads

data = gravity_loads(0.0, -0.180)  # data['T_L'], data['T_R'], data['F_y']

8. Week 1–3 is now complete

Literature notes

CAD sketch and frozen dimensions

Inverse kinematics, four assemblies, workspace

Unbalanced T_w and the force-gauge prediction F at E

Week 4–6 starts GSM: spring-torque model, targeted configuration, k and ψ. Do not change d, l1, l3, m_E unless a later print requires a uniform scale.